https://www.youtube.com/watch?v=0BJqISKuKmE VEDIO solar design system.

https://www.sunnydesignweb.com/sdweb/#/Home  Solar design system

https://pvwatts.nrel.gov/   pay back calculator

https://www.wholesalesolar.com/solar-information/start-here/offgrid-calculator home appliances calculator.

https://www.altestore.com/store/calculators/on_grid_calculator/ ON GRID SYSTEM

https://youtu.be/ahjZ4QwSxTg   batteries calculation

http://www.lauritzen.biz/solutions/calculator.html    solar pump calculation

الأحتياج الفعلي اليومي = الأحتياج اليومي÷ فعالية المحول

مجموع قوة الألواح المطلوب بالوات = مجموع الأستهلاك اليومي بالوات ÷ عدد ساعات الشمس الدنيا ÷ فعالية النظام

حساب مقاس بنك البطاريات بالوات = الأحتياج اليومي الفعلي × الأيام الأحتياطي × حرارة البطارية ÷ نسبة الأفراغ

حساب مقاس بنك البطاريات بالأمبير الساعي = مقاس بنك البطاريات بالوات ÷ نظام الفولت (فولت اللوح)

عدد البطاريات في كل مجموعة تسلسلية = نظام الفولت (فولت اللوح) ÷ فولت البطارية

عدد المجموعات التسلسلية = مقاس بنك البطاريات بالأمبير الساعي ÷ حجم البطارية

حساب عدد البطاريات = عدد البطاريات في كل مجموعة تسلسلية × عدد المجموعات التسلسلية

حساب قوة المنظم بالأمبير = ISC ×عدد الالواح التسلسلية × معدل السلامة (1.56 )

حساب قوة المنظم بالفولت = VOC ×عدد الالواح المتوازية × معدل السلامة(1.56)